CONTROL4

HOME AUTOMATION SYSTEM

Installation & Configuration Guide

2026 Edition

For Certified Control4 Integrators & Advanced Smart Home Professionals

Covers: OS X4  |  Composer Pro  |  Hardware Installation  |  Full-System Commissioning

Version 2.0  |  May 2026

Confidential — For Authorized Integrators Only

# Table of Contents

# Chapter 1: Introduction

## 1.1  What Is Control4 — And Why It Is Different

Control4 is a professional-grade, closed-ecosystem home automation platform built for integrators. Unlike consumer-level systems such as Tuya, Smart Life, or Google Home, Control4 is not available directly to end-users — it must be installed, programmed, and maintained by a Certified Control4 Dealer. This fundamental difference defines everything about the platform: its reliability, its depth of customization, and its long-term value.

### Control4 vs. Tuya / Smart Life — Key Differences

| Feature | Control4 | Tuya / Smart Life |
| --- | --- | --- |
| Distribution | Authorized dealers only | Consumer retail / DIY |
| Programming | Composer Pro (professional software) | Mobile app, limited logic |
| Reliability | Local processing, no cloud dependency | Cloud-dependent, subject to outages |
| Integration Depth | Thousands of certified drivers | Limited, mostly Wi-Fi devices |
| Scalability | 200+ rooms, 100s of devices | Typically small homes, basic needs |
| Latency | <100ms local response | 500ms–3s via cloud |
| Voice Control | Native Alexa, Google, Siri Shortcuts | Third-party, unreliable |
| Support | Dealer + Control4 corporate support | Community forums only |
| Warranty | 3-year hardware + dealer support | Varies, often none |
| Price Point | Premium ($5,000–$500,000+ projects) | Budget ($50–$2,000 projects) |

NOTE

Coming from Tuya? The single most important mindset shift is this: Control4 is a SYSTEM, not a collection of devices.

Every device, room, and scene is part of one unified project file — the .c4p project — and everything

is programmed in one place: Composer Pro. There are no separate apps per device.

## 1.2  Target Users for This Guide

This guide is written for:

- Experienced smart home installers transitioning from Tuya/Smart Life or other consumer platforms to Control4

- Junior Control4 technicians working under a Certified Dealer on their first full residential project

- IT professionals who have been hired to support Control4 infrastructure (networking, VLANs, PoE)

- Project managers who need to understand the technical process to communicate with clients and contractors

## 1.3  Prerequisites

### Skills Required

- Intermediate electrical knowledge (reading wiring diagrams, working with 120/240V circuits)

- Structured cabling experience (CAT6, HDMI, speaker wire, RS-232)

- Basic networking knowledge (IP addressing, subnets, VLANs, PoE switches)

- Familiarity with Windows-based software (Composer Pro runs on Windows only)

- Experience with audio/video systems (amplifiers, matrix switches, displays)

### Tools Required

- Laptop running Windows 10/11 (for Composer Pro)

- Control4 Composer Pro license (obtained through your dealer account)

- Network cable tester and punch-down tool

- Multimeter and non-contact voltage tester

- Drill, screwdrivers, and cable staple gun

- Label maker (every cable MUST be labeled)

- USB-A to USB-B cable (for some controller configurations)

- Laptop with direct Ethernet port or USB-to-Ethernet adapter

DANGER

ELECTRICAL SAFETY: Never work on live circuits. Always verify power is OFF with a

non-contact voltage tester before touching any wiring. Control4 lighting panels carry

120/240V AC. Failure to de-energize circuits can result in serious injury or death.

Comply with all local electrical codes. In most jurisdictions, panel work requires a

licensed electrician.

KEY TAKEAWAYS

Control4 is a dealer-only, professional platform — it cannot be self-installed by consumers.

Coming from Tuya/Smart Life, expect a steeper learning curve but far more powerful results.

You need Composer Pro (Windows) and a dealer account before you can do anything.

Every device lives in one unified project file — think of it as the 'brain' of the whole house.

Electrical safety is paramount — always kill power and verify before touching wiring.

# Chapter 2: Pre-Installation Phase

## 2.1  Site Survey Checklist

A thorough site survey is the foundation of every successful Control4 project. Never skip this step — surprises discovered during installation cost far more time and money than surprises found during the survey. Budget at least half a day for a 4-5 bedroom home.

### Physical Infrastructure

- Identify location for main equipment rack (typically in garage, IT closet, or AV room)

  - Minimum recommended rack space: 12U for a 4-bedroom home

  - Ensure rack area has dedicated 20A circuit, adequate ventilation, and cable access

- Walk every room and confirm conduit/cable pathway availability

- Identify existing cabling — CAT5e/6, speaker wire, coaxial — and test all runs

- Note ceiling height and wall construction (drywall, concrete, plaster) for cable routing

- Locate all electrical panels and confirm adequate breaker slots for lighting modules

- Identify door and window locations for sensor placement

- Note outdoor areas requiring weather-rated devices (pool, patio, outdoor speakers)

### Network Infrastructure

- Confirm ISP connection type and speed (minimum 100Mbps recommended)

- Verify router supports VLAN tagging (or plan to replace)

- Count total IP devices expected: touchscreens, cameras, door stations, audio endpoints

- Plan VLAN architecture: IoT VLAN, AV VLAN, Security VLAN, Management VLAN

- Identify PoE switch location(s) and power budget requirements

IMAGE PLACEHOLDER

[Insert Image: Site survey floor plan with cable pathway markings and equipment locations]

### Audio/Video Assessment

- Map all TV locations, projector rooms, and outdoor display areas

- Identify source locations: cable boxes, Apple TVs, Blu-ray players, gaming consoles

- Note audio zones: living room, kitchen, bedrooms, outdoor, pool

- Confirm intercom or doorbell video requirements

- Document any existing AV equipment the client wants to retain and integrate

## 2.2  Client Requirements Gathering

Before designing the system, conduct a formal needs analysis meeting with the client. Use the following framework to capture their requirements:

### Daily Lifestyle Questions

- What is your morning routine? (Lights turning on, blinds opening, music starting, coffee maker?)

- How do you use your home in the evening? (Arrival scenes, dinner modes, movie night automation?)

- Do you have children? (Consider parental controls on AV, restricted hours for lights/TV)

- Do you travel frequently? (Vacation mode, security automation, remote monitoring)

- What is your comfort level with technology? (Some clients want one-button simplicity; others want full control)

### Control Preferences

- Preferred interfaces: Keypads, touchscreens, mobile app, voice, or all of the above?

- Room-by-room control preferences — which rooms need dedicated touchscreens vs. keypads?

- Do they want a centralized 'Welcome Home' or 'Goodnight' button?

- Integration with existing devices: alarm system brand, HVAC brand, gate/intercom brand

PRO TIP

Always ask clients to describe their IDEAL scenario, not the technology. Ask: 'When you walk into

your bedroom at night, what happens?' then design the automation to match. Clients who describe

their ideal scenario give you better requirements than clients who try to specify technology.

## 2.3  System Design Best Practices

### Design Principles

- Always design for ZONES, not individual devices — a zone is a logical group (Bedroom 1, Kitchen, etc.)

- Design for the wife test: if the spouse cannot operate the system without assistance, it has failed

- Build in future expansion — always leave spare ports, breakers, and conduit

- Document everything in CAD/Visio before ordering a single piece of equipment

- Use consistent naming conventions throughout: no 'Light 1', 'Light 2' — use 'Master Bedroom Ceiling'

### Naming Convention Standard

| Entity | Format | Example |
| --- | --- | --- |
| Rooms | Floor_RoomName | GF_LivingRoom, FF_MasterBed |
| Lighting | RoomName_FixtureType | LivingRoom_Ceiling, Kitchen_UnderCabinet |
| AV Sources | SourceType_Location | AppleTV_LivingRoom, BluRay_MediaRoom |
| Keypads | RoomName_KP_Location | Entry_KP_Front, Kitchen_KP_Main |
| Scenes | Verb_Scene | Good Morning, Movie Night, Away |

## 2.4  Bill of Materials (BOM) — 4-5 Bedroom Luxury Home

The following is a realistic BOM for a high-end 4-5 bedroom residential project. Prices are dealer cost estimates and will vary:

### Controllers & Core Infrastructure

| Item | Model | Qty | Purpose |
| --- | --- | --- | --- |
| Primary Controller | Control4 CA-10 | 1 | Main system controller, up to 100 connections |
| Secondary Controller | Control4 EA-3 | 1 | AV matrix control and secondary processing |
| PoE Switch | Pakedge S16E (or Luxul) | 2 | Network backbone, PoE for touchscreens |
| Router/Firewall | Pakedge RK-1 or Luxul XR-600 | 1 | VLAN routing, remote access |
| Wi-Fi Access Points | Pakedge WK-2 (or Araknis) | 4-6 | Whole-home Wi-Fi coverage |

### User Interfaces

| Item | Model | Qty | Notes |
| --- | --- | --- | --- |
| 7" Touchscreen | Control4 T4 Series (TS-7) | 3 | Main living areas, master bedroom |
| 10" Touchscreen | Control4 T4 Series (TS-10) | 1 | Entry/foyer statement piece |
| 5-button Keypad | Control4 KPZ-5-EM | 8 | Standard room keypads |
| 7-button Keypad | Control4 KPZ-7-EM | 4 | Lighting control in larger spaces |
| In-Wall Tablet | Control4 T4-10 PoE | 1 | Kitchen command center |

### Lighting Control

| Item | Model | Qty | Notes |
| --- | --- | --- | --- |
| Dimmer (Single Pole) | Control4 Adaptive Phase Dimmer | 24 | Main room dimmers |
| Dimmer (3-Way) | Control4 3-Way Dimmer Kit | 8 | Staircase and hallways |
| Switch (Relay) | Control4 Relay Switch | 12 | Fans, non-dimmable loads |
| Outlet Dimmer | Control4 2-Button Outlet Dimmer | 6 | Lamp circuits |
| Outdoor Dimmer | Control4 Outdoor Switch | 4 | Landscape and exterior lighting |

### Audio / Video Distribution

| Item | Model | Qty | Notes |
| --- | --- | --- | --- |
| 4K HDMI Matrix | WyreStorm MX-0808-HDBT | 1 | 8x8 4K60 with HDBaseT outputs |
| Streaming Audio | Sonos Amp | 6 | Whole-home audio zones |
| In-Ceiling Speakers | Episode ES-700 Series | 24 | 6 stereo pairs + mono zones |
| Outdoor Speakers | Episode OD-700 8" | 4 | Patio and pool areas |
| Subwoofer | Sonance Sub10D | 2 | Media room and living room |

### Security & Access

| Item | Model | Qty | Notes |
| --- | --- | --- | --- |
| IP Camera (Interior) | Hikvision DS-2CD2T47G2 | 6 | Interior corridors and entries |
| IP Camera (Exterior) | Hikvision DS-2CD2T87G2 | 8 | Perimeter cameras |
| Video Doorbell | Control4 DS2 Door Station | 2 | Front and service entrances |
| Smart Lock | Schlage Encode Plus | 4 | All external doors |
| Motion Sensors | DSC PG9914 (C4 compatible) | 8 | Key rooms and exterior |

### Climate Control

| Item | Model | Qty | Notes |
| --- | --- | --- | --- |
| Smart Thermostat | Control4 TC-1 Wireless | 4 | One per HVAC zone |
| Motorized Shades | Somfy Glydea 35 DCT | 16 | Living areas and master |
| Shade Controller | Control4 SDZ-S48 | 2 | Powers motorized shades |

WARNING

Always add 15-20% contingency to BOM quantities for custom lighting projects.

Wiring runs are rarely as estimated. Order extra cable, connectors, and terminations.

Confirm all driver availability in the Control4 Driver Database BEFORE ordering third-party equipment.

Not all brands have official Control4 drivers — verify at drivers.control4.com.

KEY TAKEAWAYS

A site survey is non-negotiable — never skip it, no matter how small the project seems.

Gather client requirements by asking about LIFESTYLE, not technology preferences.

Use consistent naming conventions from day one — renaming 50 devices mid-project is painful.

Always verify Control4 driver availability for ALL third-party equipment before purchasing.

Build in 15-20% contingency in your BOM — installations always have surprises.

# Chapter 3: Hardware Installation

## 3.1  Installing the Controllers

### Overview of Control4 Controllers (2026)

Control4 offers several controllers at different performance tiers. Choosing the right controller is critical — undersizing the controller is the most common mistake new integrators make.

| Controller | Connections | Best For | Key Features |
| --- | --- | --- | --- |
| CA-1 | 15 connections | Apartments, small homes | Entry-level, Zigbee, 4 USB, HDMI |
| CA-10 | 100 connections | Mid-size homes (3-5 beds) | Quad-core, Zigbee, 4K passthrough |
| EA-1 | 15 connections | Secondary rooms/controller | Compact, PoE-powered, AV capable |
| EA-3 | 30 connections | Multi-zone AV control | 6-zone audio, Zigbee, serial/IR |
| EA-5 | 100 connections | Large estates (6+ beds) | Octa-core, full AV, max automation |

### Step-by-Step: Installing the CA-10 / EA-5 Controller

- Mount the rack shelf at the appropriate height in the equipment rack (1U for CA-10, 2U for EA-5).

- Route and terminate the Ethernet connection: use CAT6 with RJ-45 terminations, connect to your IoT/AV management VLAN port on the PoE switch.

- Connect HDMI passthrough cable if using the controller's HDMI output for main TV.

- Connect USB keyboard/mouse if performing initial network setup locally.

- Connect IR emitters if controlling IR-only devices: plug into the IR Out ports (usually 4 ports), route emitters to each IR device.

- Connect RS-232 serial cables if required (for matrix switches, processors): use the DB-9 serial ports and appropriate null-modem or straight-through cable as required by the driver.

- Secure all cables with cable ties and label every connection.

- Connect power: plug the included power supply into the IEC connector, then into a UPS-protected outlet.

- Power on: the controller will boot in 60-90 seconds. The LED will cycle through colors before settling on solid white (normal operation) or solid blue (setup mode).

IMAGE PLACEHOLDER

[Insert Image: CA-10 controller mounted in rack with all connections labeled — front and rear views]

NOTE

Always connect controllers to a UPS (Uninterruptible Power Supply). Power fluctuations and

outages can corrupt the controller's database, requiring a full restore from backup.

Minimum recommended UPS: APC Back-UPS 1500VA or equivalent.

### Network Configuration for Controllers

- On first boot, the controller broadcasts a Wi-Fi network named 'Control4-[serial]'. Connect your laptop to this network.

- Open a browser and navigate to 192.168.1.1. The Network Configuration page will load.

- Set a STATIC IP address within your management VLAN range (example: 10.10.10.10/24, gateway 10.10.10.1).

- Enter DNS servers (use your router IP + 8.8.8.8 as secondary).

- Enable Remote Access under Settings > 4Sight if you have a 4Sight subscription.

- Apply settings — the controller will reboot and connect to your main network.

- Verify connectivity by pinging the controller's IP from your laptop on the same VLAN.

## 3.2  Installing Touchscreens (T4 Series)

### T4 Series Overview

The T4 Series is Control4's flagship touchscreen line for OS X4. Available in 7-inch (TS-7) and 10-inch (TS-10) variants, these PoE-powered displays offer a premium glass interface and are designed for flush wall mounting or countertop/tabletop stands.

### Installation Steps — In-Wall Flush Mount

- Cut the wall opening using the provided template (typically 80mm x 140mm for TS-7). Use a drywall saw and be careful of any wiring in the wall.

- Pull the CAT6 cable through the opening. Leave at least 30cm of slack inside the wall.

- Terminate the CAT6 cable with an RJ-45 connector using T568B wiring standard.

- Connect to a PoE+ (IEEE 802.3at) port on your switch — the T4 draws 15.4W and requires 802.3at, NOT 802.3af.

- Insert the mounting bracket into the wall opening and secure with drywall anchors.

- Snap the touchscreen onto the mounting bracket — you will hear a click when properly seated.

- The touchscreen will boot automatically within 45 seconds.

- On first boot, you will see the Control4 setup screen — the screen will self-discover the controller on the same VLAN within 2-3 minutes.

IMAGE PLACEHOLDER

[Insert Image: T4 Series TS-7 touchscreen mounting bracket and wall cutout dimensions diagram]

WARNING

VERIFY PoE BUDGET before installing multiple touchscreens. A TS-10 draws up to 25.5W.

A 16-port PoE+ switch typically has a 185W total budget. Calculate total draw:

Example: 3x TS-7 (15.4W) + 2x TS-10 (25.5W) = 97.2W — well within budget for cameras and APs.

Exceeding the PoE budget causes random device reboots and intermittent failures.

## 3.3  Installing Keypads

### Control4 Keypad Variants

- KPZ-5-EM: 5-button, flush mount, Z2IO wireless, standard electrical box

- KPZ-7-EM: 7-button, flush mount, ideal for lighting control

- KPZDI-7-EM: 7-button with LED dimmer slider built in

- Note: All modern Control4 keypads use Z2IO (Zigbee 2 / IO) wireless — no wired data needed

### Installation Steps

- At the electrical box location, ensure a single-gang low-voltage bracket is installed (keypads do NOT connect to line voltage — they are low-voltage devices).

- Route a CAT5e/6 cable from the keypad location back to the controller or PoE switch if using the wired version (KPZ-W models). For wireless keypads, only power is needed.

- Snap the keypad clip assembly into the single-gang bracket.

- For wired models: terminate CAT5e to the RJ-45 connector on the back of the keypad.

- For wireless models: insert the battery pack (3x AA Energizer Lithium recommended — do NOT use alkaline, which may leak).

- Attach the button face — align the top and snap in. Choose button label engravings at time of order or use blank buttons with LED backlighting for a clean look.

- Apply the color finish frame (available in multiple finishes to match electrical plate styles).

IMAGE PLACEHOLDER

[Insert Image: KPZ-5-EM keypad exploded view showing button assembly, clip, bracket, and finishing frame]

PRO TIP

Order button labels with the keypad at time of purchase — laser-engraved labels look far

more professional than printed stick-on labels and clients expect them in a luxury install.

Typical label engravings: 'ALL OFF', 'GOOD MORNING', 'MOVIE', 'AWAY', 'WELCOME HOME'.

## 3.4  Lighting Control Installation

### Dimmer and Switch Installation

Control4 dimmers and switches are line-voltage devices (120V/240V) and must be installed by — or with the supervision of — a licensed electrician. They replace standard wall switches in a standard electrical box.

DANGER

DANGER — LIVE VOLTAGE: Control4 dimmers carry 120/240V AC on their terminals.

ALWAYS turn off the circuit breaker AND verify with a non-contact voltage tester

before handling wiring. Working on live circuits is illegal and potentially fatal.

Comply with NEC, IEC, or your local electrical code at all times.

### Single-Pole Dimmer Installation Steps

- Turn off the circuit breaker for the circuit you are working on.

- Verify power is off with a non-contact voltage tester at the switch location.

- Remove the existing switch and photograph the existing wiring before disconnecting anything.

- Identify the LINE (hot from panel), LOAD (going to fixture), NEUTRAL (white), and GROUND (green/bare) wires.

- Connect the wires to the Control4 dimmer terminals: LINE to Line, LOAD to Load, NEUTRAL to Neutral, GROUND to ground screw.

- Note: NEUTRAL is REQUIRED for Control4 dimmers — they do not work on 2-wire (hot+load only) circuits. If no neutral exists, a neutral must be pulled.

- Gently fold the wires and push the dimmer into the box. Secure with the provided screws.

- Apply the Control4 color frame and button face (must match keypad finish for consistency).

- Restore power. The dimmer LED will blink during initial Zigbee pairing mode.

- From Composer Pro, run a Zigbee device scan to discover the dimmer (see Chapter 4).

IMAGE PLACEHOLDER

[Insert Image: Control4 adaptive phase dimmer wiring diagram — single-pole and 3-way configurations]

### 3-Way Dimmer Installation

For staircase and hallway lighting with two switch points:

- Install the primary dimmer at the primary control location (same as single-pole above).

- At the secondary location, install the Control4 Accessory Dimmer (also called the Remote Dimmer). This unit carries no load — it communicates wirelessly to the primary dimmer.

- Wire the accessory dimmer to Line and Neutral only — do NOT connect a load.

- The two dimmers pair automatically over Zigbee within the Control4 system — no traveler wire needed in a typical retrofit.

WARNING

Load compatibility matters: Control4 dimmers are rated for LED, CFL, and incandescent loads.

Always confirm the dimmer is listed for the load type. Using an incandescent dimmer on a large

LED load can cause buzzing, flickering, and premature device failure.

Minimum load: most dimmers require at least one bulb (25W equivalent) to operate correctly.

## 3.5  Audio/Video Distribution

### HDMI Matrix Switch Installation

The AV matrix switch is the heart of the video distribution system. In a typical 4-5 bedroom project, an 8x8 HDBaseT matrix (e.g., WyreStorm MX-0808-HDBT) distributes 4K60 video to up to 8 rooms.

- Mount the matrix switch in the rack (typically 2U). Ensure 1U of airspace above and below for ventilation.

- Connect all HDMI sources to the inputs: label each input (Input 1: Apple TV, Input 2: Cable Box, etc.)

- Run CAT6/CAT6A cables from the rack to each TV location. CAT6 supports HDBaseT up to 70m; CAT6A supports up to 100m.

- At each TV location, install the HDBaseT receiver (typically a small box behind the TV): connect the CAT6 cable to the receiver's input and the HDMI output to the TV's HDMI port.

- Connect the RS-232 or Ethernet control port from the matrix switch to the Control4 controller or to the network switch.

- Connect control IR emitters to the matrix switch if required for input switching.

- Power all components and verify HDMI signal pass-through with a test source before commissioning.

IMAGE PLACEHOLDER

[Insert Image: AV rack layout diagram showing matrix switch, Sonos amps, and source devices with cable routing]

### Sonos Audio Distribution

Sonos Amp is the most popular choice for Control4 whole-home audio integration due to its native driver and rock-solid reliability.

- Mount Sonos Amps on a rack shelf or in a designated AV closet. Leave adequate airspace — Sonos Amps run warm.

- Connect left and right speaker terminals to the in-ceiling speakers. Use 16AWG minimum for runs under 30m; use 14AWG for longer runs.

- Connect the Sonos Amp to the network via Ethernet (preferred over Wi-Fi for rack-mounted amps).

- In the Sonos app, perform initial room naming setup. Use the same room names you will use in Control4.

- In Composer Pro, add a Sonos driver and configure room-to-Sonos mapping (see Chapter 4).

## 3.6  Network Infrastructure Requirements

### VLAN Architecture for a Control4 Home

Proper VLAN segmentation is essential for security and reliability. The following architecture is a professional best practice:

| VLAN | ID | Subnet | Devices |
| --- | --- | --- | --- |
| Management | 10 | 10.10.10.0/24 | Controllers, switches, router, APs |
| AV / IoT | 20 | 10.10.20.0/24 | Touchscreens, keypads, Sonos, TVs, matrix |
| Security | 30 | 10.10.30.0/24 | IP cameras, door stations, NVR |
| Client Wi-Fi | 40 | 192.168.40.0/24 | Client phones, tablets, laptops |
| Guest Wi-Fi | 50 | 192.168.50.0/24 | Guest devices — fully isolated |

NOTE

INTER-VLAN ROUTING RULES: Allow Management VLAN to reach all other VLANs (for Composer Pro access).

Allow Client Wi-Fi VLAN to reach AV/IoT VLAN (for mobile app control).

Block Security VLAN from all outbound internet access except NTP.

Block Guest VLAN from ALL internal VLANs — internet access only.

### PoE Switch Configuration

- Access your managed switch via its IP address (or console port on first setup).

- Create VLANs 10, 20, 30, 40, 50 as tagged VLANs on the uplink port to the router.

- Assign access ports (untagged) to the appropriate VLAN per device:

  - Ports 1-4: VLAN 10 (controllers and switch management)

  - Ports 5-12: VLAN 20 (AV devices and touchscreens)

  - Ports 13-16: VLAN 30 (cameras and door stations)

- Enable PoE on all ports that power Control4 touchscreens, cameras, and door stations.

- Enable spanning tree protocol (STP) on all ports to prevent loops.

- Configure QoS: mark voice/video traffic (DSCP AF41) as high priority.

## 3.7  Third-Party Device Integration

### Integration Summary

| Device Type | Integration Method | Key Consideration |
| --- | --- | --- |
| Sonos | Native IP driver (cloud + local) | Same network VLAN as controller |
| Lutron RadioRA 3 | IP driver via Lutron bridge | Requires Lutron Homeworks Integration |
| Samsung/LG TVs | IP driver (TV must be on same VLAN) | Disable energy-saving auto-off |
| Schlage Encode | Z-Wave or IP driver | Use Z-Wave version for local control |
| Hikvision Cameras | RTSP + NVR driver | ONVIF Profile S compatibility needed |
| Alexa / Google | Control4 Official Skill/Action | Requires active 4Sight subscription |
| HVAC (Ecobee) | Official IP driver | Account linking required in driver setup |
| Motorized Shades (Somfy) | RS-232 or IP via SDN Gateway | SDN gateway needed for bidirectional |

PRO TIP

Before writing a single line of custom programming for a third-party device, ALWAYS check

drivers.control4.com and DriverCentral.io first. There are over 20,000 available drivers.

Buying a $30 driver saves 10+ hours of custom development time. Never reinvent the wheel.

KEY TAKEAWAYS

Always use STATIC IPs for all Control4 core components — controllers, matrix switches, Sonos Amps.

CAT6 is the minimum standard for all data runs; use CAT6A for HDBaseT runs over 50 meters.

VLAN segmentation is not optional on a professional install — it is mandatory for security and reliability.

NEUTRAL wire is required for all Control4 dimmers — verify this before ordering dimmers.

Verify all third-party driver availability at drivers.control4.com BEFORE installing the hardware.

Always add a UPS on the equipment rack — power blips corrupt controller databases.

# Chapter 4: Programming with Composer Pro

## 4.1  Setting Up Composer Pro

### System Requirements

- Operating System: Windows 10 or Windows 11 (64-bit) — Mac is NOT supported

- RAM: 8GB minimum, 16GB recommended for large projects

- Storage: 20GB free disk space minimum

- Network: Must be on the same VLAN as the Control4 controller for direct connection

- Composer Pro Version: Always install the version matching your controller's OS version

### Installation Steps

- Download Composer Pro from your Control4 Dealer Portal at dealer.control4.com.

- Run the installer as Administrator. Accept the default installation path.

- Install the required Microsoft .NET framework and Visual C++ redistributables if prompted.

- Launch Composer Pro after installation. On first launch, it will prompt for your dealer credentials.

- Enter your Dealer Portal username and password. This activates your license.

- Composer Pro will load and show the 'Start' screen with options to connect to a controller or create a project offline.

IMAGE PLACEHOLDER

[Insert Image: Composer Pro main screen layout showing System Design pane, Properties pane, and Programming pane]

## 4.2  Project Creation and Backup Best Practices

### Creating a New Project

- In Composer Pro, click File > New Project. A wizard will guide you through initial setup.

- Enter the project name using your naming convention (e.g., 'Smith_Residence_2026').

- Select the controller model that matches the hardware installed on-site.

- Click Connect to Controller to establish a live connection, OR choose Work Offline to pre-program before arriving on site.

### Backup Best Practices

- Save a local backup every time you make significant changes: File > Save Project As > save to a date-stamped folder

- Use cloud backup: File > Upload to 4Sight — this enables remote restore if the controller is replaced

- Naming convention for backups: ProjectName_YYYY-MM-DD_Rev01.c4p

- Keep at least 3 versions: previous version, last working version, and current version

- Before any major programming session: backup FIRST

WARNING

NEVER work without a recent backup. A corrupted project file after a power loss or

network dropout can cost you hours of reprogramming. The 30 seconds to save a backup

before starting work can save an entire day of effort.

Store backups in at least two locations: your laptop AND a cloud drive (Dropbox/OneDrive).

## 4.3  Creating Rooms, Devices, and Drivers

### Building the System Design

The System Design pane in Composer Pro is the foundation of your project. Everything — rooms, devices, connections — is set up here before any programming happens.

### Step 1: Create Rooms

- In System Design, right-click on 'Rooms' in the left panel > Add Room.

- Name the room using your naming convention (e.g., 'Living Room', 'Master Bedroom').

- Assign the room to a floor level (Ground Floor, First Floor, etc.).

- Set the room's default audio/video source (can be changed later).

- Repeat for every room in the project. A 4-bedroom home typically has 12-18 rooms including hallways, outdoor areas, and utility rooms.

### Step 2: Add Devices and Load Drivers

- Right-click on a room > Add Device. The Device Browser window opens.

- Search for your device by manufacturer and model number. Example: search 'Sonos Amp' to find the official Sonos driver.

- Double-click the driver to add it to the room. The driver's properties will appear in the Properties pane.

- Enter required configuration: IP address, port number, serial number (varies per driver).

- For Zigbee devices (dimmers, keypads): right-click the device > Identify, then press the button on the physical device to pair it.

- Repeat for all devices in every room.

IMAGE PLACEHOLDER

[Insert Image: Composer Pro System Design view showing room hierarchy with devices listed under each room]

### Step 3: Connections — Linking Devices

Connections tell Composer Pro how devices relate to each other. This is what separates Control4 from simple Zigbee mesh systems.

- Navigate to the Connections pane (View > Connections).

- The left panel shows all devices; the right panel shows connection points.

- Example: Link the AV Matrix Switch output to a TV in the Living Room — drag the 'HDMI Out 1' connection from the matrix switch to the 'HDMI In' on the Living Room TV device.

- Example: Link a Sonos Amp to the Living Room audio zone — connect the Sonos Amp device to the Living Room room object.

- Example: Link a camera to the Video Doorbell to the front door room entry point.

PRO TIP

Think of Connections like physical cable routing on paper. If you would run an HDMI cable

between two physical devices, you need a Connection between them in Composer Pro.

Connections tell the system's UI what sources and destinations are available in each room.

## 4.4  Advanced Programming

### Understanding the Programming Model

Control4 uses an event-driven programming model. The fundamental concept is:

NOTE

WHEN [Event] IF [Condition] THEN [Action]

Example: WHEN Front Door Keypad Button 1 is pressed

IF Time is between 8:00 AM and 10:00 PM

THEN Turn on Entry Lights, Unlock Front Door, Show Front Camera on Living Room TV

### 4.4.1  Variables

Variables store state information that can be used in conditions and actions.

- Boolean Variable: True/False — example: 'GuestMode' (On/Off)

- Number Variable: 0–65535 — example: 'OccupancyCount' (how many people are home)

- String Variable: Text — example: 'LastActivatedScene' (name of last scene run)

#### Creating a Boolean Variable:

- In Programming, click Variables in the left panel.

- Click Add > Boolean Variable. Name it 'VacationMode'.

- Set the default value to FALSE.

- You can now use 'VacationMode = TRUE' as a condition in any event, or set it from a keypad button, schedule, or mobile app toggle.

### 4.4.2  Conditions and Logic

Conditions allow you to check state before executing actions. Multiple conditions can be combined with AND / OR logic.

#### Example — Intelligent Arrival Scene:

- Event: Front door lock unlocked

- Condition 1: Time is between 6:00 PM and 11:00 PM (AND)

- Condition 2: Variable 'SomeoneHome' = FALSE

- Action 1: Set Variable 'SomeoneHome' = TRUE

- Action 2: Turn on Entry Foyer lights at 70%

- Action 3: Turn on Kitchen lights at 50%

- Action 4: Set HVAC to Comfort mode

- Action 5: Play 'Arrival' music playlist on Sonos at 30% volume

- Action 6: Display front door camera on foyer touchscreen for 10 seconds

### 4.4.3  Timers and Delays

Timers allow you to execute actions after a period of time or at a specific interval.

#### One-Shot Timer Example — Auto-Off for Bathroom Fan:

- Create a new Timer: Programming > Timers > Add Timer. Name it 'BathFanTimer', set to 30 minutes, one-shot.

- Event: Bathroom Fan Switch turned ON

- Action: Start BathFanTimer

- Event: BathFanTimer fires

- Action: Turn OFF Bathroom Fan Switch

- Event: Bathroom Fan Switch turned OFF

- Action: Stop BathFanTimer (in case user manually turns it off)

#### Delay Example — Staged Lighting:

To create a cinematic staged lighting effect when arriving home:

- Action 1: Turn on Entry Lights at 40% — Delay 1.5 seconds

- Action 2: Turn on Living Room Lights at 30% — Delay 1.5 seconds

- Action 3: Turn on Kitchen Lights at 50% — Delay 1.5 seconds

- Action 4: Turn on Dining Room Lights at 40%

IMAGE PLACEHOLDER

[Insert Image: Composer Pro programming screen showing event tree with conditions, delays, and action commands]

### 4.4.4  Scenes and Routines

Scenes are predefined lighting and device states that can be activated by a button press, schedule, or automation. They are the most frequently used feature in a Control4 system.

#### Creating a 'Movie Night' Scene:

- In System Design, right-click on Living Room > Add Scene > Name it 'Movie Night'.

- In the scene editor, add the following state changes:

  - Living Room Ceiling Light: 0% (off)

  - Living Room Accent Lights: 15% (dim warm glow)

  - Kitchen Lights: 20% (ambient)

  - AV Matrix Switch: Select Input 1 (Apple TV) to Living Room TV output

  - Living Room TV: Power On

  - Sonos Living Room: Set volume to 25%, select movie soundtrack playlist

  - Motorized Shades: Close all living room shades

  - HVAC: Set to 22°C (comfort mode)

- Save the scene. Now assign it to a keypad button: in Programming, find the keypad button event > Action: Activate Scene 'Movie Night'.

### 4.4.5  Complex Automation Logic Examples

#### Example 1: Occupancy-Based HVAC Control

- Devices needed: Motion sensors in each room, thermostat

- Logic: WHEN last motion in any room exceeds 30 minutes: set HVAC to Eco mode

- Logic: WHEN first motion detected after Eco mode: return to Comfort mode

- Logic: WHEN Vacation Mode variable = TRUE: set HVAC to Vacation mode (manual override)

#### Example 2: Intelligent Good Morning Routine

- Trigger: Schedule at 7:00 AM on Weekdays (Monday-Friday)

- Condition: Variable 'SomeoneHome' = TRUE

- Action 1: Ramp Master Bedroom lights from 0% to 30% over 15 minutes

- Action 2: After 5 minutes: open master bedroom motorized shades

- Action 3: After 10 minutes: start Sonos Morning Playlist at 20% in kitchen

- Action 4: After 12 minutes: turn on kitchen under-cabinet lights at 80%

- Action 5: After 15 minutes: set HVAC to Morning Comfort temperature

#### Example 3: Security Integration — Panic Response

- Trigger: Alarm system sends 'Panic' event to Control4 driver

- Action 1: Turn ALL lights in ALL rooms to 100% simultaneously

- Action 2: Turn on ALL exterior lights

- Action 3: Lock ALL smart locks

- Action 4: Display ALL camera feeds on entry touchscreen and master bedroom touchscreen

- Action 5: Send push notification to all family members via 4Sight

IMAGE PLACEHOLDER

[Insert Image: Complex programming tree showing nested conditions, variables, and multi-room actions for Good Morning routine]

### 4.4.6  Voice Control Integration

Control4 OS X4 supports native integration with Amazon Alexa, Google Assistant, and Apple Siri Shortcuts without requiring third-party middleware.

#### Amazon Alexa Setup:

- Ensure the controller has an active 4Sight subscription (required for voice integration).

- In Composer Pro, go to Settings > 4Sight > Voice Services > Enable Alexa.

- On the client's Alexa app, search for the 'Control4' skill and enable it.

- Link their 4Sight account credentials in the Alexa skill login page.

- Alexa will discover all Control4 devices that have been marked as 'Voice Controllable' in Composer Pro.

- In Composer Pro, mark devices for voice control: right-click each device > Properties > Enable Voice Control > set the Alexa-friendly name (e.g., 'Living Room Lights').

NOTE

Voice Control Naming Tips: Use simple, distinct names that are easy to say.

Avoid names like 'Light 1' or 'Zone A'. Use: 'Living Room Lights', 'Master Bedroom',

'Kitchen TV'. Alexa and Google work best with 2-4 word names.

Test every device name by actually saying it aloud — if it feels awkward, rename it.

### 4.4.7  Notifications and Alerts

Control4 4Sight push notifications can be triggered by any programmable event.

#### Common Notification Examples:

- 'Front door unlocked with code 1234' — sends when cleaning staff arrives

- 'Water leak detected at kitchen sensor' — sends immediately on sensor trigger

- 'Garage door left open for 15 minutes' — prevents accidentally leaving garage unsecured

- 'Pool gate opened while in vacation mode' — security alert for unattended access

- 'Generator started — power outage detected' — system awareness

#### Setting Up a Push Notification:

- In Programming, find the event that should trigger the notification (e.g., Water Leak Sensor: Wet event).

- Add Action > Communication > Push Notification.

- Select which 4Sight accounts receive the notification (can be multiple family members).

- Enter the notification message text. Keep it clear and actionable: 'WATER LEAK: Kitchen sensor activated — check under sink immediately.'

- Optional: add a Repeat action to resend every 5 minutes until acknowledged.

KEY TAKEAWAYS

Always BACKUP before programming — a 30-second save prevents hours of lost work.

The programming model is WHEN > IF > THEN — master this pattern and you can build anything.

Use Variables to build smart, state-aware automations (not just simple button mappings).

Scenes are your most powerful client-facing feature — invest time making them perfect.

Test every automation in every possible scenario — check edge cases like late-night arrival.

Voice control names must be tested by speaking them aloud — not just looking good on screen.

# Chapter 5: Control4 OS X4 Configuration

## 5.1  Overview of OS X4

Control4 OS X4 (pronounced 'X4') is the latest generation of the Control4 operating system. Introduced in 2023 and significantly updated through 2025-2026, OS X4 delivers a completely redesigned user experience with a tile-based interface, improved app performance, and deeper third-party integration.

### Key Features of OS X4

- Redesigned tile-based home screen with customizable room cards

- Improved media browsing with album art and streaming service integration

- Native Matter protocol support (Thread and Wi-Fi devices)

- Enhanced intercom and camera viewer

- Improved energy monitoring dashboard

- Faster UI response — typically under 100ms for lighting commands

## 5.2  Setting Up the X4 Interface

### Updating to OS X4

- In Composer Pro, go to Help > Software Update. If OS X4 is available for your controller model, it will appear here.

- Read the release notes carefully before updating — some older drivers may need updates for X4 compatibility.

- Click Update. The controller will download the OS update (ensure it has internet access) and reboot. This process takes 5-15 minutes.

- After reboot, reconnect Composer Pro and verify all devices are responding.

- Check the Control4 driver compatibility matrix for any drivers needing updates.

WARNING

OS UPDATES: Never update controller OS during an active client commissioning session.

Always update on a test system first, or update the day BEFORE going on-site.

Keep a backup project file saved BEFORE updating — rare rollback situations do occur.

OS X4 requires minimum firmware version 3.4.0 on all touchscreens — update UIs before OS update.

### Customizing the X4 Home Screen

- In Composer Pro, navigate to Interfaces > Screen Saver and UI Editor.

- For each touchscreen, configure the room cards that appear on the home screen — drag to reorder.

- Set the default room for each touchscreen (typically the room the touchscreen is located in).

- Upload a custom background image (optional but recommended for luxury projects). Resolution: 2560x1440px for TS-10, 1280x800px for TS-7.

- Configure the screensaver: set timeout (recommended 3 minutes), choose from clock, weather widget, or custom image slideshow.

- Set the wake-on-proximity feature if the touchscreen has a proximity sensor (TS-10 has this built in).

IMAGE PLACEHOLDER

[Insert Image: Control4 OS X4 home screen on TS-10 touchscreen showing room tiles, climate widget, and media controls]

## 5.3  Mobile App Configuration

### Control4 for iOS and Android

The Control4 app is the primary mobile interface. It provides full system control and mirrors the touchscreen UI.

### App Setup Steps

- Ask the client to download 'Control4' from the App Store (iOS) or Google Play Store (Android).

- Open the app and tap 'Create Account' if they do not have a 4Sight account, or 'Sign In' if they do.

- Account setup requires an email address and password — document this for the client handover.

- On the home screen, tap the '+' icon and select 'Add System'. The app will search for Control4 systems on the local Wi-Fi network.

- The system will appear in the list (it broadcasts via mDNS). Tap to connect.

- If on the same Wi-Fi network as the controller, the app connects locally. On remote networks, it connects via 4Sight cloud — requires an active subscription.

- Test both local and remote connectivity before client handover.

PRO TIP

Set up the client's phone on-site before leaving. Walk through the app with them.

Show them: how to control lights, change sources, adjust volume, arm/disarm, run scenes.

Clients who feel confident in the app within the first 5 minutes are far more satisfied with the system.

## 5.4  User Accounts and Permissions

### Account Tiers

- Owner Account: Full access to all features, can manage other users, can access Composer Express

- User Account: Can control devices and scenes, cannot change system settings

- Guest Account: Limited access, typically lighting and audio only, time-restricted access available

### Creating User Accounts

- In Composer Pro, go to Settings > Accounts > User Management.

- Click Add User. Enter the user's name, email address, and role (Owner/User/Guest).

- Set allowed rooms — for a nanny account, you might restrict access to children's rooms and kitchen only.

- Set time restrictions if needed: 'This account can only access the system between 7:00 AM and 8:00 PM'.

- An invitation email is automatically sent to the user's email address with app setup instructions.

NOTE

SECURITY BEST PRACTICE: The dealer account (your Composer Pro account) provides full

programming access and should never be shared with clients. Create a separate 'Owner'

account for the client. The dealer retains programming access via 4Sight remote connection.

## 5.5  Remote Access Setup

### 4Sight Remote Access

4Sight is Control4's remote access and cloud services subscription. It provides remote programming access for dealers and remote control for users.

- Verify the controller has a 4Sight license activated (check in Composer Pro: Settings > 4Sight Status).

- Ensure the controller can reach the internet on port 443 (HTTPS) and port 6000 (Control4 proprietary) — confirm with the router/firewall configuration.

- Test remote access: disconnect your laptop from the client's Wi-Fi (use your phone as a hotspot) and reconnect Composer Pro using the 4Sight connection option.

- Verify remote access response time — typically 3-8 second connection establishment, then local-speed control.

- Document remote access credentials securely in your company's password manager.

KEY TAKEAWAYS

OS X4's tile-based UI is the client's daily interface — invest time making it beautiful and intuitive.

Always test BOTH local and remote app connectivity before client handover.

User accounts and permissions protect the client's privacy — a nanny should not access the master bedroom.

4Sight is required for remote access, voice control, and push notifications — ensure it is active.

Document all account credentials securely — you will need them for every future service call.

# Chapter 6: Testing & Commissioning

## 6.1  Full System Testing Checklist

Commissioning is the systematic process of verifying that every device, scene, and automation works correctly. Never skip steps — even if you are confident in your programming. Every experienced integrator has been surprised by something that 'should have worked'.

### Network and Infrastructure

- Verify all controller IP addresses are static and pingable from management VLAN

- Verify all touchscreens are reachable and show correct room assignment

- Verify all Sonos devices are discoverable and assigned to correct rooms in Control4

- Confirm internet access works from controller (required for 4Sight and driver updates)

- Test VLAN isolation: Client Wi-Fi cannot reach Security VLAN (try pinging camera from phone)

- Verify PoE budget — no devices should be rebooting randomly

### Lighting Control

- Test every single dimmer and switch — turn on and off from both physical button AND app

- Verify dimming range: 0%-100% with no flicker at any level

- Test all 3-way dimmer pairs — primary and accessory both control the lights

- Verify all scenes activate correctly — check every device state change in each scene

- Test schedule-based events: set a test schedule 2 minutes in the future and observe

- Test all keypad buttons — every button should have a programmed function

### Audio/Video

- Test every HDMI source to every display — all 8x8 or 4x4 matrix combinations

- Verify 4K/HDR pass-through on matrix switch for all 4K sources

- Test audio in every room — verify left/right channel balance, volume control

- Test AV follow-me: if enabled, verify audio follows client between rooms

- Verify Sonos grouping and ungrouping works from Control4 interface

- Test video doorbell: press doorbell, verify screen pop-up on all touchscreens and mobile app

### Security and Access

- Test every smart lock: lock and unlock from keypad, mobile app, and scheduled automation

- Verify all cameras display in Control4 camera viewer (no black screens)

- Test motion sensors: walk in front of each sensor and verify trigger event fires in Composer Pro

- Test panic button if installed: verify all lights turn on and notifications are sent

- Test doorbell video: visitor presses doorbell, verify call rings on touchscreen and mobile app

### Climate and Shades

- Verify HVAC control: change temperature setpoints from Control4 app and verify thermostat responds

- Test every motorized shade: open and close from app, keypad, and automation

- Verify shade position feedback if shades support bidirectional communication

- Test 'Good Morning' automation including shade opening sequence

IMAGE PLACEHOLDER

[Insert Image: System commissioning test sheet template — printed checklist for on-site sign-off]

## 6.2  Common Issues and Troubleshooting

| Symptom | Likely Cause | Solution |
| --- | --- | --- |
| Dimmer flickering at low levels | LED incompatibility or minimum load | Check dimmer LED compatibility list; adjust minimum trim in dimmer settings |
| Touchscreen shows wrong room | Incorrect room assignment in Composer Pro | In System Design, check touchscreen's room assignment; update and sync |
| Zigbee device not pairing | Out of Zigbee mesh range or interference | Move device closer to controller for pairing; add Zigbee mesh repeaters if needed |
| Sonos not responding | IP address conflict or VLAN routing issue | Set Sonos to static IP; verify AV VLAN can reach Composer Pro laptop |
| TV not turning on via Control4 | IP driver timeout or TV sleep mode | Set TV to never auto-power-off in TV settings; check IP driver connection |
| Remote access not working | 4Sight subscription lapsed or firewall block | Check 4Sight status in dealer portal; verify port 443 and 6000 open outbound |
| Push notifications not arriving | 4Sight not linked to user account | Re-link 4Sight account in user settings; check app notification permissions |
| Schedule not firing | Controller timezone mismatch | In Composer Pro: Settings > Controller > confirm correct timezone and NTP sync |
| Motorized shades not responding | RS-232 cable polarity issue | Verify RS-232 straight-through vs null-modem; check baud rate in driver settings |
| Matrix switch not switching | Control IP not in same VLAN | Verify matrix switch IP is static and reachable from controller VLAN |

## 6.3  Client Training Process

Client training is as important as the technical installation. A client who does not understand their system will call you constantly — or worse, stop using it and tell everyone the system is 'too complicated'.

### Training Session Structure (90 minutes recommended)

- Start with the BIG PICTURE (5 minutes): Show the client the system overview — 'This is your controller, this is your app, these are your keypads. They all work together.'

- Daily scenes walkthrough (20 minutes): Walk through the scenes they will use every day: Good Morning, Good Night, Welcome Home, Movie Night. Let THEM press the buttons — don't just show them, have them do it.

- App demonstration (20 minutes): Walk through the mobile app on their phone: control lights, change music, check cameras, run scenes. Make sure they can do it without assistance by the end.

- Keypad training (15 minutes): Walk room by room and explain every keypad button. Place a printed keypad guide in each room (see Section 7.2).

- Voice control (10 minutes): Demonstrate Alexa/Google commands. Give a quick-reference card with example commands.

- FAQ and Q&A (20 minutes): Ask: 'What would you like your system to do that we have not shown you?' Address any questions and make notes for follow-up programming tweaks.

PRO TIP

Video record your client training session (with their permission). This gives you proof that

you trained them if they later claim they 'were never shown' something. It also serves as a

reference for the client if they forget how to do something — share the video recording with them.

KEY TAKEAWAYS

Commission every single device individually before testing scenes and automations.

Use a printed checklist during commissioning — sign off each item, do not rely on memory.

Common issues: Zigbee range problems, LED dimmer incompatibility, VLAN routing gaps.

Client training is NOT optional — a well-trained client is a satisfied client and a referral source.

Record client training sessions — protects you legally and gives clients a reference to replay.

# Chapter 7: Handover & Documentation

## 7.1  Client Handover Package

A professional handover package demonstrates that you operate at a different level than a DIY installer. It builds trust, reduces support calls, and creates a foundation for a long-term service relationship.

### Handover Package Contents

- System Overview Document: One-page summary of all installed equipment with photos

- Network Diagram: Printed network topology with IP addresses, VLANs, and device names

- Equipment Inventory: Full list of all devices with model numbers, serial numbers, and locations

- Keypad Function Guide: Per-room laminated card showing every button's function

- Scene Library: Description of every scene and how to activate it

- Quick Start Card: One page, large font — the 5 most common things they will do daily

- App Download Instructions: QR codes for iOS and Android app stores

- Emergency Contact Card: Your company's 24/7 support number and email

- Warranty Information: What is covered, for how long, and how to make a claim

- Login Credentials Card (sealed envelope): All account usernames for the client's records

IMAGE PLACEHOLDER

[Insert Image: Example professional client handover folder with branded materials and laminated guides]

## 7.2  Keypad Function Reference Cards

Create a laminated A6 card for each room that has a keypad. The card lives in a pocket behind the keypad or in the card holder (available as a Control4 accessory). Sample format:

| Button | Name | Function |
| --- | --- | --- |
| Button 1 (Top) | GOOD MORNING | Ramp lights to day mode, open shades, start morning playlist |
| Button 2 | MOVIE NIGHT | Dim lights to 15%, close shades, start Apple TV, set surround |
| Button 3 | DINNER | Kitchen 80%, dining 60%, soft music, close outdoor shades |
| Button 4 | ALL OFF | Turn off all lights in this room |
| Button 5 (Bottom) | GOOD NIGHT | Turn off all lights in house, lock all doors, set alarm |

## 7.3  Maintenance and Future Expansion

### Regular Maintenance Schedule

- Monthly: Log into Composer Pro remotely and verify all devices show as online

- Quarterly: Check for OS updates and driver updates — install with client's pre-approval

- Quarterly: Review push notification logs for any recurring errors or device failures

- Annually: Physical inspection of rack equipment, cable connections, and touchscreens

- Annually: Review and update scenes based on client's evolving lifestyle needs

### Planning for Future Expansion

NOTE

EXPANSION READINESS: At the time of initial installation, pull spare conduit to any room

that might get AV equipment in the future (home office, guest bedroom, outdoor areas).

Leave spare ports on the matrix switch, spare zone capacity on Sonos Amps, and spare

Zigbee capacity on the controller. Control4 systems grow over time — plan for this.

- Document all spare ports, conduit stubs, and available capacity in the system overview

- Inform the client about platform expansion possibilities: outdoor cinema, home office AV, EV charger integration, energy management

- Control4 can integrate with: Crestron systems (via drivers), KNX bus systems, Modbus industrial controls — useful for mixed commercial/residential projects

## 7.4  Warranty and Support Information

### Control4 Hardware Warranty

- Standard warranty: 2 years for controllers and touchscreens; 1 year for lighting control

- Extended warranty: available through the dealer for up to 5 years

- Warranty registration: register all equipment at dealer.control4.com within 30 days of installation

### Dealer Service Agreements

A recurring service agreement benefits both the dealer and the client:

- Remote Monitoring Plan: Dealer monitors system health monthly — detects issues before client notices

- Priority Support Plan: Guaranteed same-day or next-day response for any system issues

- Annual Review Plan: Includes one site visit per year for system review and driver updates

- Typical pricing: $500-$2,500/year depending on system complexity and services included

PRO TIP

Service agreements are your most valuable recurring revenue stream. A 200-client service

agreement portfolio at $1,000/year average = $200,000 guaranteed annual revenue.

Pitch the service agreement at handover when the client is most excited about their system.

Frame it as 'peace of mind' — not a technical product. Clients pay for the outcome, not the SLA.

## 7.5  Final Sign-Off

### Project Completion Documentation

- Complete the Commissioning Checklist (Section 6.1) and file a signed copy.

- Obtain the client's signature on the system acceptance form.

- Upload the final project backup to 4Sight cloud.

- Store a copy of the project file in your company's project archive with client name and date.

- Update the project in your CRM system with all device serial numbers and installed locations.

- Send the client a follow-up email 2 weeks after installation: 'How is the system working for you? Any questions?'

KEY TAKEAWAYS

A professional handover package elevates you above competitors — spend time making it polished.

Laminated keypad guides in every room reduce support calls dramatically.

Always pull extra conduit and leave spare capacity — Control4 systems always grow.

Register all warranty within 30 days of installation — protect the client and yourself.

Service agreements create recurring revenue — pitch them at handover while enthusiasm is high.

Follow up 2 weeks post-installation — one extra hour of your time generates referrals and loyalty.

# Appendices

## Appendix A — Quick Reference: Default Port Numbers

| Protocol / Service | Port | Direction | Purpose |
| --- | --- | --- | --- |
| Control4 4Sight | 443 (HTTPS) | Outbound | Remote access and cloud services |
| Control4 4Sight | 6000 (TCP) | Outbound | Composer Pro remote connection |
| Control4 mDNS | 5353 (UDP) | Local LAN | Device discovery |
| Sonos API | 1400 (TCP) | Local LAN | Control4 Sonos driver |
| RTSP (Cameras) | 554 (TCP/UDP) | Local LAN | Camera video streams |
| RS-232 via IP | 4999 (TCP) | Local LAN | Serial-over-Ethernet converters |
| Lutron HomeWorks | 23 (Telnet) | Local LAN | Lutron integration driver |
| SDDP Discovery | 1902 (UDP) | Local LAN | Control4 device auto-discovery |

## Appendix B — Zigbee Mesh Design Guidelines

- Minimum Zigbee mesh coverage: one mesh device (dimmer, switch, or dedicated repeater) per 10m of linear distance

- Maximum recommended hops between any end-device and controller: 4 hops

- Concrete walls significantly reduce Zigbee range — expect 30-40% range reduction per wall

- Use Zigbee network tools in Composer Pro (System > Zigbee Network Map) to visualize mesh coverage

- Battery-powered Zigbee devices (keypads, sensors) are endpoint devices — they DO NOT repeat signal

- Wired Zigbee devices (dimmers, switches) act as routers — they DO repeat signal

## Appendix C — Recommended Driver Sources

- Control4 Driver Database: drivers.control4.com — official Control4 certified drivers

- DriverCentral: drivercentral.io — largest third-party driver marketplace, subscription model

- Chowmain Software: chowmain.biz — premium drivers for niche devices

- Black Wire Designs: blackwiredesigns.com — AV and security specialist drivers

- Custom Driver Development: use Control4's Lua SDK for bespoke integrations

## Appendix D — Useful Keyboard Shortcuts in Composer Pro

| Shortcut | Action |
| --- | --- |
| Ctrl + S | Save project |
| Ctrl + Z / Y | Undo / Redo |
| Ctrl + Shift + S | Save project as (backup) |
| F5 | Refresh device discovery |
| Ctrl + D | Duplicate selected item |
| Ctrl + F | Find device or driver |
| Ctrl + Shift + P | Open Programming pane |

## Appendix E — Glossary

| Term | Definition |
| --- | --- |
| 4Sight | Control4's cloud subscription service enabling remote access, voice control, and push notifications |
| BOM | Bill of Materials — the full equipment list for a project |
| Composer Pro | The Windows-based programming software used to configure Control4 systems |
| Driver | Software plugin that enables Control4 to control a third-party device |
| HDBaseT | Technology for transmitting 4K HDMI video over CAT6/6A cable up to 100 meters |
| OS X4 | Control4 Operating System generation 4 — latest as of 2026 |
| PoE | Power over Ethernet — delivers DC power through CAT5e/6 cable alongside data |
| VLAN | Virtual Local Area Network — logical network segment for device isolation |
| Zigbee | Low-power mesh wireless protocol used by Control4 dimmers, keypads, and sensors |
| Z2IO | Control4's enhanced Zigbee implementation with improved range and security (2024+) |

─── End of Document ───

Control4 Home Automation System — Installation & Configuration Guide (2026 Edition)

For Certified Control4 Integrators — Confidential